| REQUEST FOR QUOTATION (RFQ) |  | Supplier’s Stamp/Signature: |
| --- | --- | --- |
| Closing Date | : 05 November 2025 |  |
| Email Submission | : box8@belts.com.bn |  |
| BELTS Ref | : BELTS/HCCS/RFQ/2025/027 |  |
| Date of Submission | : Click here to enter text. |  |
| Supplier/Quote Ref | : Click here to enter text. |  |

| Company Name | Click here to enter text. |
| --- | --- |
| Address | Click here to enter text. |
| Phone | Office: Click here to enter text. Mobile: Click here to enter text. |

| No. | Item Description | Delivery Period | Qty/UoM | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| The contractor shall carry out Preventive Maintenance (PM) services for all motorised roller shutters, totaling eight (8) units, installed at BELTS MSC warehouse. The preventive maintenance shall be conducted quarterly over a period of one (1) year. The objective of this Preventive Maintenance is to ensure that all roller shutters operate safely, smoothly, and reliably at all times, thereby minimizing downtime and extending the equipment’s operational lifespan. |  |  |  |  |  |
| 1. | PREVENTIVE MAINTENANCE (PM) SERVICES FOR EIGHT (8) UNITS MOTORIZED SHUTTERS AT BELTS MSC WAREHOUSE SCOPE OF WORK: - Motor & Drive System Check motor condition and operation; inspect gearbox, coupling, and chain drive; lubricate moving parts; test manual override function. | 7-10 WORKING DAYS UPON PO ISSUANCE & PTW APPROVAL | 4 AU | Click here to enter text. | Click here to enter text. |
|  | - Control Panel Inspect wiring, fuses, relays, limit switches, and control buttons; test up/down/stop functionality; check safety interlocks. |  |  |  |  |
|  | - Roller Drum & Shaft Inspect for alignment, wear, and unusual vibration; clean and lubricate bearings. |  |  |  |  |

| No. | Item Description | Delivery Period | Qty/UoM |  | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Guide Rails & Brackets Inspect for wear or damage; ensure proper alignment; clean and lubricate. | 7-10 WORKING DAYS UPON PO ISSUANCE & PTW APPROVAL |  |  |  |  |
|  | - Safety Devices Test safety edge sensors, photocells, and limit switches for correct function. |  |  |  |  |  |
|  | - Electrical Connections Tighten loose connections; check for burnt or corroded terminals. |  |  |  |  |  |
|  | - Operational Test Perform full up/down cycle test; check for smooth operation, abnormal sound, or jerky movement. |  |  |  |  |  |
|  | Note: - The address is at: - BELTS Medical Supply Chain (MSC), Spg 433, Rimba Highway, Kg. Madaras. - For any site visit request, kindly email your proposed date & time schedule to admincorp@belts.com.bn - For any questions or enquiries kindly email directly to rfq_enquiry@belts.com.bn - Contractors must implement safety measures to protect workers and public during installation/delivery work. - Vendors/contractors must adhere to regulatory SHENA safety rules and regulations. - The vendor awarded for this RFQ must submit a complete Permit To Work (PTW) document checklists requested for the work. |  |  |  |  |  |
| MANDATORY Suppliers are to provide with the Packaging Dimension, Measurement & Weight - in Standard measurement in Meter, Feet, Inches, Cm, Mm, Kg, GM |  |  |  | TOTAL PRICE |  | Click here to enter text. |
| Freight Term (EXW,CIF,ETC) |  |  |  |  |  | Click here to enter text. |
| Freight Charges (IF APPLICABLE) |  |  |  |  |  | Click here to enter text. |
| Any other charges please state |  |  |  |  |  | Click here to enter text. |
| GRAND TOTAL |  |  |  |  |  | Click here to enter text. |

| Currency Code : | BND |
| --- | --- |
| Price Validity (Minimum 9 Months) : | Click here to enter text. |
| Payment Terms : | 30 DAYS CREDIT |
| Warranty : | Click here to enter text. |
| Mode of Delivery : | Click here to enter text. |
| Delivery Period : | Click here to enter text. |
| Country of Manufactured : | Click here to enter text. |
| Remarks: | Click here to enter text. |